NAME: SANTHOSH D

REG NO: (192412209)

COURSE NAME: ANALOG AND DIGITAL COMMUNICATION (ECA0802)

EX NO: 2a

GENERATION AND ANALYSIS OF FREQUENCY MODULATED

(FM) SIGNALS USING MATLAB

Objectives

(i) To generate a Frequency Modulated (FM) signal using MATLAB by varying the frequency of

a high-frequency carrier in accordance with the amplitude of the message signal and to observe

the resulting waveform in the time domain.

(ii) To analyze the frequency spectrum of the generated FM signal using FFT and identify the

carrier component, sideband frequencies, and the bandwidth of the FM signal based on Carson's

rule.

Objective 1: To generate a Frequency Modulated (FM) signal using MATLAB by varying the

frequency of a high-frequency carrier in accordance with the amplitude of the message signal

and to observe the resulting waveform in the time domain.

PROGRAM (MATLAB CODE)

clc;

clear;

close all;

Am=1;

Ac=1;

fm=5;

fc=50;

beta=5;

kp=5;

fs=1000;

t=0:1/fs:1;

m = Am*cos(2*pi*fm*t);

c = Ac*cos(2*pi*fc*t);

% Message signal

% Carrier signal

fm_sig = Ac*cos(2*pi*fc*t + beta*sin(2*pi*fm*t)); % FM

subplot(4,1,1), plot(t,m), title('Message Signal')

subplot(4,1,2), plot(t,c), title('Carrier Signal')

subplot(4,1,3), plot(t,fm_sig), title('FM Signal')

OUTPUT:

Objective 2: To analyze the frequency spectrum of the generated FM signal using FFT and

identify the carrier component, sideband frequencies, and the bandwidth of the FM signal based

on Carson's rule

PROGRAM (MATLAB CODE)

% Calculation and Verification of Frequency Deviation

% and Modulation Index of FM Signals

clc;

clear;

close all;

%% Input Parameters

Am = 2;

% Message signal amplitude

fm = 500;

Ac = 1;

fc = 10000;

kf = 1000;

fs = 100000;

T = 0.02;

t = 0:1/fs:T;

% Message frequency (Hz)

% Carrier amplitude

% Carrier frequency (Hz)

% Frequency sensitivity (Hz/V)

% Sampling frequency (Hz)

% Simulation duration (s)

%% Message Signal

m = Am*cos(2*pi*fm*t);

%% Frequency Deviation Calculation

delta_f = kf * Am;

%% Modulation Index Calculation

beta = delta_f/fm;

%% FM Signal Generation

fm_signal = Ac*cos(2*pi*fc*t + beta*sin(2*pi*fm*t));

%% Plot Message Signal

figure;

subplot(2,1,1);

plot(t,m,'LineWidth',1.5);

title('Message Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

%% Plot FM Signal

subplot(2,1,2);

plot(t,fm_signal,'LineWidth',1.5);

title('Frequency Modulated Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

%% Spectrum Analysis

N = length(fm_signal);

FM_fft = fftshift(abs(fft(fm_signal))/N);

f = (-N/2:N/2-1)*(fs/N);

figure;

plot(f,FM_fft,'LineWidth',1.5);

title('FM Signal Spectrum');

xlabel('Frequency (Hz)');

ylabel('Magnitude');

grid on;

xlim([fc-10000 fc+10000]);

%% Display Results

fprintf('\nFM SIGNAL PARAMETERS\n');

fprintf('---------------------------\n');

fprintf('Carrier Frequency

= %.0f Hz\n',fc);

fprintf('Message Frequency

fprintf('Message Amplitude

= %.0f Hz\n',fm);

= %.2f V\n',Am);

fprintf('Frequency Sensitivity  = %.0f Hz/V\n',kf);

fprintf('\nCALCULATED VALUES\n');

fprintf('---------------------------\n');

fprintf('Frequency Deviation (Δf) = %.0f Hz\n',delta_f);

fprintf('Modulation Index (β)     = %.2f\n',beta);

%% Carson's Rule Bandwidth

BW = 2*(delta_f + fm);

fprintf('Bandwidth (Carson Rule) = %.0f Hz\n',BW);

OUTPUT: