NAME: SANTHOSH D

REG NO: (192412209)

COURSE NAME: ANALOG AND DIGITAL COMMUNICATION (ECA0802)

EX NO: 3a

GENERATION OF PHASE MODULATED (PM) SIGNALS AND

CALCULATION OF PHASE DEVIATION USING MATLAB

SIMULATION

Objective

i) To generate a Phase Modulated (PM) signal in MATLAB using specified carrier and message

signal parameters.

ii) To observe and analyze the time-domain characteristics of the message signal, carrier signal,

and phase modulated signal through MATLAB simulation.

iii) To calculate the phase deviation of the PM signal using the phase sensitivity constant and

message signal amplitude.

iv) To determine the modulation index of the PM signal and examine its relationship with phase

deviation.

v) To investigate the effect of varying message signal amplitude on the phase deviation and

modulation characteristics of the PM signal.

vi) To compare PM waveforms obtained for different phase deviation values and analyze their

modulation behavior using MATLAB.

PROGRAM (MATLAB CODE)

clc;

clear;

close all;

% Parameters

Am = 1;

fm = 100;

Ac = 5;

fc = 1000;

fs = 20*fc;

kp = pi/2;

% Message Amplitude (V)

% Message Frequency (Hz)

% Carrier Amplitude (V)

% Carrier Frequency (Hz)

% Sampling Frequency (Hz)

% Phase Sensitivity (rad/V)

% Time Vector

t = 0:1/fs:4/fm;

% Message Signal

m = Am*sin(2*pi*fm*t);

% Carrier Signal

c = Ac*cos(2*pi*fc*t);

% Phase Modulated Signal

pm = Ac*cos(2*pi*fc*t + kp*m);

% Phase Deviation

phase_dev = kp*Am;

% PM Modulation Index

mp = phase_dev;

% Display Results

fprintf('Message Amplitude = %.2f V\n',Am);

fprintf('Message Frequency = %.0f Hz\n',fm);

fprintf('Carrier Amplitude = %.2f V\n',Ac);

fprintf('Carrier Frequency = %.0f Hz\n',fc);

fprintf('Phase Sensitivity = %.2f rad/V\n',kp);

fprintf('Phase Deviation = %.2f radians\n',phase_dev);

fprintf('PM Modulation Index = %.2f\n',mp);

% Plotting

figure('Name','Phase Modulation and Phase Deviation',...

'NumberTitle','off');

subplot(2,2,1);

plot(t,m,'LineWidth',1.5);

title('Message Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(2,2,2);

plot(t,c,'LineWidth',1.5);

title('Carrier Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(2,2,3);

plot(t,pm,'LineWidth',1.5);

title('Phase Modulated Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(2,2,4);

bar(phase_dev);

title('Phase Deviation');

ylabel('Radians');

set(gca,'XTickLabel',{'PM Signal'});

grid on;

OUTPUT: